# Electronics Documentation

### 1. Electronics Block Diagram

### 2. Components Table

| Component | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| Stepper Motor | 1 | To lift the pins from the base via a thread | The shaft provides a greater surface area to wrap the thread and lift the system with just a 90-degree turn. |
| NeoPixel Strip | 1 | To show how many pins have been knocked down per throw | Acts like a straight progress bar of how many pins have been knocked down. |
| IR Sensor | 2 | To detect gutter ball and display it on the OLED display | Detects the presence of the ball when it passes over it and hence obstructs the returning of the IR rays. |
| OLED Display | 1 | To show the numerical score for each of the throws and the total score along with strike and gutter ball detection. It also shows when the system is ready to use. | Text messages and numerical values can be easily displayed. |